# Risk Analysis of Vertical Rescue Systems

by Alan Sheehan B.E., Senior Vertical Rescue Instructor NSW SES

## Introduction

Having recently been exposed once again to the great single/double rope debate for vertical rescue systems, I decided to try to cut away some of the mythical and emotive arguments to get to some facts. Easier said than done. There are very few real facts, or even statistics available, to objectively analyze the pros and cons of either system.

The problem with the single/double rope debate is the lack of real data, and the emotive arguments used to promote the double rope cause, combined with fear of litigation, and limited understanding of some of the concepts which people believe they are applying when using double rope systems. This commonly results in people believing that double rope systems are safer than they really are!

So I decided to model two vertical rescue systems for a complete probabilistic failure analysis and then apply arbitrary, constant failure probabilities to every component. At a micro (say component) level, this is not accurate and not realistic. But at a macro (say system or assembly) level, this is about as good as we’re going to get without real data, and by way of comparison - all things being equal - is fair and objective for the purpose of determining, philosophically, what the difference is.

The systems I chose to model were the single rope technique system known in New South Wales as a V3 system, and essentially the exact same system but with the reversible safety replaced with a belay to make a single rope with belay system. These are essentially both as reasonably simple as possible.

## Terms and Nomenclature

Elegance	-	Elegance is a measure of achievement versus complexity. Normally expressed as Elegance = (Achievement)/(Complexity). Something is elegant when it achieves more with less.

Failure	-	condition when something no longer performs it’s intended function. In particular, in vertical rescue we are concerned with failures of components, systems and procedures which result in injury or death or significant risk of either.

Probability	-	The chance of some condition occurring. Usually denoted as P(condition). I.e. the chance of rolling a six on a die is 1 in 6, P(6) = 1/6 or P(6) = 0.166667.

Redundancy	-	Redundancy is having a parallel system to take over the function when one system fails.

Reliability	-	The probability that something will not fail in the time period considered. Reliability = 1 - P(failure).

Risk	-	the probability of failure, P(failure) or other undesirable consequence.

Safety	-	the concept of safety is a relative one. There are no absolutes in life when it comes to safety. When something is referred to as “safe”, it means the risks are within accepted levels.

## Assumptions

The first assumption is that the risk of failure of every single component in the system is the same. I.e. the risk of the rope failing is the same as that of a karabiner failing, and is the same as the litter attendant dying of his own accord (natural causes!) while on rope! This is a big assumption, but any other assumption is just as inaccurate without real data. However, the inaccuracy is of little consequence because we are not trying to say the risk of failure of system “A” is 1 in 1000, we are comparing a system “A” with a system “B” using the same common assumptions. The actual failure probabilities are more influenced by a rescue team’s protocols, equipment checks, SOP’s, and rigging practices, decisions, leadership and judgement than we can hope to predict using this model!

The second assumption is that each system will be rigged as simply as possible. Any additional links in the chain add to the risk of failure. The only exception to this is the (bolt on) haul system i.e. I haven’t assumed the simplest system is rigged, but it is common to both the single rope and single rope + belay systems so that doesn’t matter.

Another major assumption is that the risk of failure does not change with increasing complexity. This is perhaps the biggest limitation on this model, as will be explained in detail later. This assumption is what allows the concept of redundancy to be applied in strict terms, and as we will see in the discussion later, this is seldom the case for things like vertical rescue systems!

For the haul system I assumed a “typical” haul system of a 4:1 multiplying M.A. The model for the haul system is not accurate, as the multiplying M.A. contains an element of redundancy built in which is not easy to model. As the consequences of failure of the haul system are not all catastrophic, and the haul system is common to both rescue protocols being modeled, all components of the haul system were modeled as if they were all linked in series - simply for the sake of simplicity.

I have assumed that both the main brake and reversible brake descenders are simple devices such as whaletails or goldtails and not suitable to be included in haul systems (as say a Petzl ID may be).

I have assumed that all critical anchors are load sharing anchors, while the anchor for the haul system is a simple one. This, obviously, need not always be the case.

Knots have not been included in the model. Some may argue that knots are a separate entity from the rope with their own risk of failure and should be included, others would argue they are in integral part of the rope or sling. I have elected to omit them on the basis of keeping the model simple.

The litter attendant has been included in the system, as his death or injury during the rescue would be regarded as part of the rescue even if the cause of death or injury had nothing to do with the rescue! There is a small risk that anyone could die at any time for unforeseen reasons. In any case, the death of the litter attendant while on rope could add significant difficulties to the rescue and place the casualty’s life at risk.

Conversely, the casualty has been omitted from the model. If their time is up, their time is up, and no amount  of rescue will change it! We are only concerned with the vertical rescue systems causing the injury or death to the casualty.

The stretcher spider has been assumed to be a single component. In a vertical lift it may actually be so, however, many spiders will have four legs for a horizontal lift, and two for a vertical lift, so providing some redundancy. Despite some having four legs, they may have a common attachment loop, so without modelling a particular spider, I have opted to model the spider as a single component and ignore any redundancy.

## The model

Using the building block theory, the model has been built up from components to assemblies and then to systems, and finally to operations, such as using the whole system to lower, or to haul. A common risk of failure is assigned to each component, and the laws of probability used to calculate the risks of each assembly and system in turn.

The rules of probability are as follows:

- For components in series, the risks of failure, P(failure), are added.

- For components in parallel (i.e. redundant), the risks of failure are multiplied.

Following the model, 5 components make up a simple load sharing anchor: 2 anchors, 2 slings and 1 karabiner. Their arbitrary risk of failure is 1 in 10000 each, but assembled into a load sharing anchor, their risk of failure as an assembly is still just 1 in 10000. This is due to redundancy.

The risk of failure of the main brake is 2 in 10000, given the risk of failure of the load sharing anchor as well as the descender in series.

At the other end of the load line, we find the stretcher attendant tie in is triply redundant - with two ascenders and the rope tail all tying the litter attendant to the rope.

The litter attendant’s harness has been modeled as legs loops and waist belt as all good quality harnesses are built in such as way that if either fails or is cut, the remainder is adequate for life support.

For the reversible safety, again a load sharing anchor is modeled, and all other items are in series.

The haul system uses a simple anchor as it never supports the load without a safety, and this offers a comparison between a simple and load sharing anchor.

The belay system again uses a load sharing anchor. The belay chosen to be modeled is a load releasing hitch and tandem prusik belay. For the sake of the model and comparison, when the belay is used, I have assumed it replaces the reversible safety.

## Interpretation of the Results

### Anchors

The simplest comparison is between the load sharing anchor and the simple anchor. Intuitively, most people would say the risk of a load share failing would be half that of a simple anchor due to redundancy. In fact it is one third based on our model. But in reality, this not always the case, as other factors come into play - see below.

### Safety v Belay

Another interesting comparison is to compare the reversible safety to the belay. Here they both come up with the same risk of failure. But as always, there are other factors to consider and these are discussed further below.

### Single Rope Versus Single Rope + Belay

Comparison of results show that during a main line lower, the single rope + belay system has ½ the risk of failure of a single rope system, according to the theory of our model. Of course, there are other practical considerations which affect this as discussed below.

Similarly during hauling operations, the risk of failure of the single rope + belay is 2/3 of that of the single rope system.

There is no real comparison for lowering on the reversible safety in the single rope + belay system, but interestingly, the risk of failure during this operation is, in theory, 83% higher than when hauling, and 37% higher than when performing a loadline lower. This difference is due entirely to the extra components in the chain.

## Other Factors

### Complexity

Complexity is the single most evil thing that can get amongst a vertical rescue system. Every extra item is something else that must be correctly applied, checked and monitored throughout the rescue operation. Every extra item is something else to fail. The reliability of a human being to make a correct decision is about 90% when unstressed, but it quickly deteriorates to 40% when stressed! That’s one wrong decision in ten when unstressed and 6 wrong ones out of 10 when stressed.

When “the pressure is on”, a simple system which is easy to build, easy to check and monitor, is quicker to assemble and operate and results in less stress on the individual and less risk of a bad decision. Frequently rescuers are under stress when rigging and operating vertical rescue systems. Air craft maintainers, for example, are not, and this is an important point to keep in mind when it comes to the reliability of actions and decisions, combined with complexity and the redundancy debate.

The flow on effects of complexity cannot be understated. The redundancy effect due to using parallel items in the system is reduced by having more items to assemble, check, and monitor - there is a risk that not all will be assembled, checked and monitored properly. So complexity undermines redundancy.

Follow the KISS principle - Keep It Simple, Stupid!

### Redundancy

Redundancy is the buzz word that proponents of double rope systems love. Unfortunately, few really understand the limitations of the concept. Redundancy never delivers the full theoretical benefits. Interactions between the redundant components often decrease the actual benefits of redundancy, and do outside influences on common characteristics.

For example, in a pumping installation two pumps sitting side by side, one duty pump and one on standby. If the standby pump is not turning all the time, vibration from other machines (maybe the duty pump!) often results in brinelling of the bearings, so shortly after startup of the standby pump, the bearings will fail. If the pump is on a hot standby duty (i.e. it is kept turning) often the bearings are damaged by a bearing failure in the duty pump - as each revolution impacting the bearing damage sends an impulse to the standby pump at exactly the same time that the rolling elements in the standby pump’s bearings are in the same position! So the damage is transmitted to the standby pump like a disease!

Redundancy works best in permanent installations and assemblies, where the cost of down time is extreme and the service life of components is moderate to high.

Take the aircraft industry for another example. Everyone knows that redundancy is widespread in aircraft especially large commercial aircraft. They commonly use three of four parallel systems for redundancy. How could three systems possibly all fail together? When an outside influence affects a common characteristic of the three systems. Consider the Sioux City air crash - they crashed because they lost all three hydraulic circuits at the same time. How? A defect in the casting of the turbine rotor of the motor in the tailplane (smaller than the size of a grain of sand) was not detected, and subsequently caused the rotor to fail. It flew apart and cut all three hydraulic circuits where they met in the tailplane.

Vertical Rescue systems are not like aeroplanes or permanent industrial installations. Redundancy is used in aeroplanes and industry because once it is installed it is there for years! The installed life is comparable to, or longer than, the expected time to failure of the item. In a vertical rescue system, the system is built once, checked, used once, and dismantled and stowed for next time. In between jobs, all the gear should be inspected and checked. The installed life of a vertical rescue system component is usually a few hours - very much less than the expected time to failure of the item! I.e. we will not wear out an item from perfect condition to failure in one vertical rescue if we are doing the right things (we might damage a rope if we aren’t vigilant with edge protection!).

So we need to weigh the safety benefits of redundancy up against the adverse safety effects of complexity, and take into account that the theoretical benefits of redundancy, such as we have calculated in this model, will never be fully achieved!

### Load Effects

The model assumes that all loads are equal regardless of the operation being performed. This is clearly not the case.  Friction tends to increase the loads applied at/to the haul system during hauling operations, and reduce the load on the brake descenders during lowering operations. The amount of load in any component does have a direct relationship to the probability of failure of the component.

Shock loading has a far greater effect. Say, for example, the haul system anchor fails during a hauling operation...

In the single rope system, the rescue load is dropped until the reversible safety takes up. This is less than a fall factor zero fall, as the reversible brake line is extended but not loaded, but all the remainder of the loadline is also available to absorb the shock. The resulting shock forces will be considerably less than 2 times the static load depending on how long the load line is.

In the single rope plus belay system, the belay line should also be extended but with no slack, but in this case there is no shock absorbing assistance from the mainline. The fall factor will always be around zero, which will produce a dynamic load of twice the static load (this is a special case in dynamics and commonly taught to all engineers studying dynamics). The distance of the fall will be greater than with a single rope and reversible safety (i.e. 2 times the static stretch in the whole belay rope, versus 2 times the static stretch in the reversible safety line). In addition, the belay rope must go directly over the edge and down the face, where the reversible safety line does not. There is a considerable risk during the dynamic loading of the belay that the line of the rope will shift off any edge protection used, and/or will contact sharp edges down the face while under high dynamic loading further adding risk of failure of the belay rope.

So we see that load effects, both direct and indirect, can decrease the theoretical benefits of using a belay.

## Conclusion

The argument that a single rope + belay system is significantly safer than a single rope system is not conclusively proven. There are pros and cons for each.

The important aspects are:

- Simplicity is the key to safety in Vertical Rescue. Every piece of equipment is another piece to fail, to assemble, to check, to operate and to monitor. The fewer links, overall, in the system, the fewer the number of decisions to make and actions to take, so the safer it will be, especially when rescuers are under stress.

- Redundancy is not a guarantee against failure. The difference in risk between a single rope system and a single rope + belay system is not large, and indeed depends more on the individual team and the operational situation than on the protocol chosen.

- The model used here, gives the theoretical, relative risks of failure of components, assemblies and system for some pretty broad assumptions. The actual risks will differ from team to team, depending on their protocols, training, teamwork, leadership, quality of checks, experience, attitude, and a host of other factors as well.

- Regardless of the protocol chosen, single rope or single rope + belay, there is comparable risk. Indeed, if the risks are unacceptable, the rescue should not proceed. It is difficult to see circumstances where a belay is significantly safer than a single rope system (apart from for a team with a poor safety record). There may be some, but most cases where the risk to the loadline is so great that one would consider a belay, the risk to the belay line is at least as high. Such cases are playing russian roulette either way, but the belay does give a second chance - not a guarantee!

- Vertical Rescue is about acceptable risk - as is every other activity in life. The difference between the two systems considered here is marginal. Arguments that a belay makes a single rope system safe firstly, assumes that a single rope system is unsafe, and secondly, assumes that there is a significant reduction in risk with the belay. There is not.

Alan Sheehan B.E.

Senior Vertical Rescue Instructor

Oberon Unit, New South Wales State Emergency Service.