Material Safety Data Sheet.

Hematite MSDS Number 450-30

Section 1: Composition / Information on Ingredients. Product Name; Hematite is a natural mineral from Egypt. Synonyms; anhydrous iron oxide, English red, ferric oxide, Indian red, iron (III) oxide, iron sesquioxide, jeweler's rouge, Mars red, red iron oxide, red iron trioxide, red ochre, rouge, specular iron. Colour Index; Pigment Red 101. Component Name; CAS No.; % Composition Range; OSHA PEL mg/m3; ACGIH TLV mg/m3; DFG-MAK mg/m3. Chemical Formula; TWA; STEL; TWA; STEL; TWA; STEL. Ferric Oxide, Iron Sesquioxide Fe2O3; 1309-37-1; Varies; 10; N.E.1; 5; N.E.; 1.52; N.E... Section 2: Hazards Identification. Emergency Overview; Emergency Overview No unusual fire or spill hazard. Low health risk by inhalation.. Potential Health Effects. Eyes; May cause mechanical irritation.. Skin; None. Inhalation; Low health risk by inhalation. Treat as a nuisance dust.. Oral; Greater than 10g/kg (rat). Section 3: First Aid Measures. After Inhalation; Remove to fresh air. If breathing is labored or stopped, give artificial respiration. Get immediate medical attention.. After Skin Contact; Wash area of skin with soap and water.. After Eye Contact; Flush eyes with plenty of water for at least 15 minutes. Seek medical attention if irritation develops or persists.. After Ingestion; If victim is conscious and alert, give large quantities of water to induce vomiting. Seek medical attention immediately.. Section 4: Fire Fighting Measures. Fire; Not considered to be a fire hazard. Not flammable.. Explosion; Not considered to be an explosion hazard.. Extinguishing Media; This material is not combustible and is not anticipated to react with commercially employed extinguishing media. Use appropriate extinguishing media for surrounding fire.. Section 5: Accidental Release Measures. Spill Procedures; Protect against identified hazards through use of prescribed personal protection equipment, proper work and hygiene practices. Limit foot and vehicular traffic to minimize mechanical agitation and dispersion. Employ a vacuum, equipped with HEPA (High Efficiency Particulate Air) filter, for clean-up of the spill material. If no vacuum is available, use a broom and shovel to collect excess powder in the area. Recover uncontaminated material for use. Vacuum or sweep remaining material keeping dust to a minimum. Residual material should then be cleared, utilizing the process of wet sweeping, to avoid dust generation.. Containment Techniques; This is a solid material and will not travel far from the spill location unless mechanically agitated.. Spill Response Equipment; The following equipment is recommended for spill response: • vacuum, equipped with a HEPA filter • broom, wet mop • dustpan, shovel, or scoop • bags, drums or sacks for collection. Personal Protective Equipment; All personnel should utilize the following protective equipment when performing spill response activities: • gloves (rubber or leather) • safety glasses or goggles • respiratory equipment as recommended in Section 8. Section 6: Handling and Storage. Storage; A moderately dry, well-ventilated area is considered adequate for handling and storage. Usual precautions for nuisance dust should be followed.. Handling; When handling this product, all personnel are directed to: • Wear all specified elements of PPE, as directed by this document, or under location specific requirements, whichever imore conservative. • Avoid creating dust, where possible.. Section 7: Exposure Controls / Personal Protection. Engineering Controls; Use with adequate ventilation to meet exposure limits listed in Section 2.. Respiratory Protection; Use NIOSH-approved dust respirator, if overexposure exists.. Skin Protection; Leather or rubber gloves.. Eye/Face Protection; Safety glasses, goggles or face shield are recommended. • To control potential exposures, avoid creating dust.. Work Hygiene Practices; Do not eat, drink, smoke, or perform other hand-to-mouth activities in product use or handling area. • Wash thoroughly after handling this product.. Section 8: Physical and Chemical Properties. Earths are natural products. Technical data varies or are not measurable.. Appearance; Physical State: Solid Color: Red to reddish brown Form: Powder. Odor; Odorless. Section 9: Stability and Reactivity. Stability; Stable under ordinary conditions of use and storage.. Hazardous Decomposition Products; None. Hazardous Polymerization Products; None. Incompatibilities; Iron oxides are not compatible with hydrazine, calcium hypochlorite, performic acid and bromine pentaflouride.. Conditions to Avoid; None. Section 10: Toxicological Information. Ingestion; There are no known dangerous acute or chronic effects associated with the use of this product. The acute oral toxLD 50 (rat) for Fe2O3 is greater than 10g/kg (rat).. Skin; No known dangerous acute or chronic effects.. Eye; If dust intrudes into the eyes, a slight eye irritation can occur.. Section 11: Ecological information. No harmful effects known other than those associated with suspended inert solids in water.. Section 12: Disposal Considerations. Recommended Disposal Method; Collect in containers, bags or covered dumpster boxes. Whatever cannot be saved for recovery or recycling should be managed in an appropriate and approved waste disposal facility. Processing, use or contamination of this product may change the waste management options. State and local disposal regulations may differ from federal disposal regulations. Dispose of container and unused contents in accordance with federal, state and local requirements.. Section 13: Transportation Information. U.S. DOT; Not regulated for domestic transport by land, rail or air. Enter the proper freight classification on the shipping documents, "MSDS Number" and "Product Name" for shipping purposes. Canadian TDG Hazard Class and PIN Not regulated. Section 14: Regulatory Information. SARA 313 Title III; Section 311/312 Hazardous Categories: None Section 313 Toxic Chemicals: None. OSHA Status; This product is not considered hazardous.. TSCA Status; Components of this product are listed in the TSCA Inventory.. California Proposition 65; Not listed. CERCLA Reportable Quantity; None. International Regulations; Canadian Ingredient Disclosure List: Components are listed. Canadian WHMIS: This material is not a controlled substance under WHMIS. European Community: This material is not subject to classification according to EEC Directive 67/548/EEC.. Section 15: Other Information. Date. 19-09-2013 The above information is believed to be correct but does not purport to be all inclusive and shall be used only as a guide. The information in this document is based on the present state of our knowledge and is applicable to the product with regard to appropriate safety precautions. It does not represent any guarantee of the properties of the product. Geo Quarry company shall not be held liable for any damage resulting from handling or from contact with the above product..